| Drawings Reviewed: (titles plus dates) | Project Name: Project Number: Review By: Date: Approved By: BCDN: |
| --- | --- |

Legend: N Not required R Required C Confirm requirements or RC Review completed and item construction detail(s) indicated on documents This checklist is intended to assist architects in a review of code requirements to be considered in the preparation and review of documents to support a permit application. A simple review of the checklist is not a substitute for a thorough review of the OBC. Modify the checklist to note applicable code requirements for the project.

- 1. INDIVIDUAL PORTABLES [CASE 1 SEE OBC A-3.9.3]						  OBC Reference(s)

|  | a | Building area: 100m2 max. Height: 1 storey Actual Area: m2 | 3.9.3.1.(1) |
| --- | --- | --- | --- |
|  | b | Distance between classrooms > 6m | 3.9.3.2.(1) |
|  | c | Fire extinguishers | 3.9.3.5 & 3.2.5.17 |
|  | d | Means of egress | 3.3 & 3.4 |
|  | e | Min. 2 egress if no F.S. for fuel-fired appliance | 3.9.3.6.(2) |
|  | f | Fuel fired appliance: fire separation exception one appliance min. 4.5m from door | 3.9.3.7.(2) |
|  | g | Washroom facilities: in the main building (WR analysis from school board) | 3.9.3.8 |
|  | h | Spatial separation from the main building [* exposing bldg. face details-main bldg.] | 3.2.3 |
|  | i | Provisions for fire fighting [ ] > 12 portables on site: | 3.9.3.4 |

- 2. GROUP – PORTABLES [CASES 2 to 5 SEE OBC A-3.9.3]

|  | a | Distance between classrooms < 6m |  |
| --- | --- | --- | --- |
|  | b | Building area (Group) = m2 | 3.9.3.1.(2) |
|  | c | “Group” classification = | 3.2.2.24 to 28 |
|  | d | Construction requirements: | 3.2.2.24 to 28 |
|  | e | Fire extinguishers = | 3.9.3.5 & 3.2.5.17 |
|  | f | Means of egress | 3.3 & 3.4 |

|  | g | Fuel-fired appliance F.S. - 1.5 hr. [Dist. < 1.5m] - 45 min. [Dist. > 1.5 m] - no fire separation [Dist > 6m] on appliance within 4.5 m from door - no fire separation (appliance has sealed combustion, max. 4 portables & appliance located within 4.5 m from door | 3.9.3.7.(2) (a) 3.9.3.7.(2) (b) 3.9.3.7.(3) 3.9.3.7.(4) |
| --- | --- | --- | --- |
|  | h | Washroom facilities: In the main building (WR analysis from School Board) | 3.9.3.8 |
|  | i | Barrier-free access | 3.9.3.9 |
|  | j | Spatial separation from the main building [*exposing bldg. face details-main bldg.] | 3.2.3 |
|  | k | Spatial separation between portables | 3.9.3.2.(2) |
|  |  | - Exceptions [Cases 3 to 5] |  |
|  | l | Fire alarm systems | 3.9.3.3 |
|  |  | - Exceptions [Cases 2 & 3] |  |
|  | m | Provisions for fire fighting | 3.9.3.4 |
|  |  | - Exceptions where maximum 12 Portables [Cases 1 to 5] |  |