| ##### Drawings Reviewed: (titles plus dates) | Project Name: Project Number: Review By: Date: Approved By: BCDN: |
| --- | --- |

Legend: N Not required R Required C Confirm requirements or RC Review completed and item Construction detail(s) indicated on documents This checklist is intended to assist architects in a review of code requirements to be considered in the preparation and review of documents to support a permit application. A simple review of the checklist is not a substitute for a thorough review of the OBC. Modify the checklist to note applicable code requirements for the project.

1. CLASS	OBC Reference(s)

|  | a | Class (A, B, indoor, outdoor) | 3.11.2.1 |
| --- | --- | --- | --- |
|  | b | Occupant load (d/2.5 + S/1.4) | 3.1.16.3.(1) & (2) |
|  | c | Exemptions for club, camp or institutions | 3.11.3.1.(2) & (3) |
|  | d | Elevated pool decks | 3.11.3.1.(11) |

2. GENERAL REQUIREMENTS

|  | a | Attach general notes and advise Health Department / copy of M.O.H letter to contractor |  |
| --- | --- | --- | --- |
|  | b | Minimum water depth > 0.75m | 3.11.3.1.(4) & (5) |
|  | c | Bottom slopes | 3.11.3.1.(6) |
|  | d | Walls vertical to within 150mm of bottom if water depth < 1.35m | 3.11.3.1.(8) |
|  | e | Ledges | 3.11.3.1.(10) |
|  | f | Pool deck | 3.11.3.1.(9) (12) to (14) |
|  | f | Pool deck width (exemption) | 3.11.3.1.(2) |
|  | h | Recessed fittings for safety buoy line at 300mm where slopes exceeds 8% | 3.11.3.1.(7) |
|  | i | Mark deck in 100mm lettering indicating depth at deep, shallow and break between gentle and deep bottom slope points: in appropriate locations indicate in words “SHALLOW AREA” “DEEP AREA” for depth 2.5m | 3.11.3.1.(24) |
|  | j | Light colour pool surfaces | 3.11.3.1.(20) |
|  | k | Notice / sign with 150mm lettering for 2.5m water depth or less to read; “CAUTION – AVOID DEEP DIVES” or “SHALLOW WATER – NO DIVING” | 3.11.3.1.(25) |
|  | l | Black disc 150mm diameter at deepest point | 3.11.3.1.(21) |

|  | m | Diving boards | 3.11.4.1 |
| --- | --- | --- | --- |
|  | n | Corresponding water depth in front of diving board | 3.11.4.1.(6) to (8) |
|  | o | Clearance above board | 3.11.4.1.(15) |
|  | p | Ladders or steps from pool | 3.11.3.1.(18) |

3. DRESSING, LOCKERS, SHOWERS AND TOILETS

|  | a | Exemption for class A&B pool alternative facilities | 3.11.9.1.(2) & (3) |
| --- | --- | --- | --- |
|  | b | Water closet, urinals, lavatories provided | 3.7.1.2.& 3.11.9.1.(4) |
|  | c | Showers (1 per 40 bathers) | 3.11.9.1.(5) |
|  | d | Showers for disabled | 3.8 |
|  | e | Showers beside pool deck | 3.11.9.1.(6) |
|  | f | Privacy provisions (dressing area, showers, etc.) | 3.11.9.1.(12) & (13) |
|  | g | Floor sloped in washroom, showers, passages | 3.11.9.1.(9) |
|  | h | Lockable access to deck | 3.11.3.1.(1B) |

4. SERVICE ROOMS AND STORAGE

|  | a | If room contains compressed chlorine gas provide gas tight 1 hr. fire separation at or above ground with outside exit door | 3.11.11.1.(2)(a) |
| --- | --- | --- | --- |
|  | b | Chemical storage facility (refer to mechanical section for ventilation, hose bib and floor drain) | 3.11.11.1.(2)(a) |

5. EMERGENCY PROVISIONS

|  | a | Emergency illumination (pool and rooms) | 3.11.10.1.(4) to (6) |
| --- | --- | --- | --- |
|  | b | Emergency telephone | 3.11.10.1.(7) & (8) |
|  | c | Emergency stop button | 3.11.10.1.(12) to (14) |

6. SPECIAL POOLS (carry out detailed Code review and inspection on the following:)

|  | a | Public pool in the B2 and B3 occupancy ramped | 3.11.5.1 |
| --- | --- | --- | --- |
|  | b | Modified pool | 3.11.6 |
|  | c | Wave action pool | 3.11.7 |